|  | MECHATRONICS DEPARTMENT TE2024 – MICROCONTROLLER LABORATORY LAB #2 – µVision 5 IDE |  |
| --- | --- | --- |
|  | Name: _________________________ ID: _________________________ | Name: _________________________ ID: _________________________ |

Instructions:

- Follow sections A, B and C (in that order).

- When writing the report, after the cover, the elements in the following sections must appear in this order:

  - C.1.18

  - C.1.21

  - C.1.22

  - C.2.5

  - C.2.6

  - C.2.7

- Remember that those points are results. Those results are the culmination of the work you do on the laboratory. Before each one of them you must answer as concise as possible the following (do not place these questions in the report):

  - What were you trying to accomplish?

  - Which obstacles you found in the way?

  - How did you sort them?

  - How your final program works?

  - What results does it throw?

  - How do you interpret the obtained results?

  - What can you conclude of the overall process?

- After you finish writing the report, name it as TE2024<X>.<Y>_E<Z>_L<W>.docx.

<X>: Bachelor ID: ISD, IMT

<Y>: Group number (not required for IMT. In this case, please ignore the dot).

<Z>: Team number

<W>: Lab number

- Create a folder named Projects\ and move inside your Keil-MDK project for each one of the final systems.

- Inside a .zip file, include both the report and the Projects\ folder.

- Upload it to the assigned place indicated by your instructor.

- APPLICATIONS SETUP

Objective: Install on at least one personal computer the required software for both theory and lab classes.

Instructions: Read and execute the steps in the following subsections.

A.1 – Keil MDK

- Run the Keil-MDK installer file as Administrator.

Figure A.1.1. Opening the Keil-MDK Installer

- Click on Next to skip the Welcome window.

Figure A.1.2. Keil-MDK Installer Welcome window

- Accept the License Agreement and click on Next.

Figure A.1.3. Keil-MDK License Agreement window

- Enter a location (preferably a folder named \TE2023\Keil_v5 at the top of the C: drive) where to install the IDE. Click on Next.

Figure A.1.4. Keil-MDK installation location

- Enter contact information and then click on Next.

Figure A.1.5. Keil-MDK customer information

- Wait until the installation is over.

Figure A.1.6. Keil-MDK Installation

- When prompted about the ULINK drivers, install none of them.

Figure A.1.7. ULINK Drivers Installation

- Untick all the options once the installation is over. Click on Finish.

Figure A.1.8. Finishing Keil-MDK Installation

- At this point, you should have an active Internet connection. When the Keil Pack Installer window prompts, click on OK. Wait until the program loads all available packages.

Figure A.1.9. Keil-MDK Pack Installer Welcome window

- Once all the packages are loaded, in the left window click in STMicroelectronics  STM32F1 Series  STM32F103.

Figure A.1.10. Keil-MDK Pack Installer Available Packages

- On the right window click on Keil::STM32F1xx_DFP, ARM::CMSIS, Keil::ARM_Compiler, Keil::MDK-Middleware in order to install them. Once done, you should have something similar to Figure A.1.11. Finally, close the window.

Figure A.1.11. STM32F103 Related Packages

- Open the Keil µVision 5 program. Set four in Edit  Configuration  Editor  C/C++ Files  Tab size. Click on OK. Then close the window.

Figure A.1.12. STM32F103 Related Packages

Figure A.1.13. STM32F103 Related Packages

A.2 – ST-LINK Utility and Drivers

- Run the STM32 ST-LINK utility installer as administrator.

Figure A.2.1. Running STM32 ST-LINK Utility installer as Administrator

- Click on Next to skip the Welcome window.

Figure A.2.2. ST-LINK Utility installer Welcome window

- Accept the License Agreement and click on Next.

Figure A.2.3. ST-LINK Utility installer License Agreement

- Enter a location (preferably in a folder named TE2023 at the top of the C: drive) where to install the IDE. Click on Next.

Figure A.2.4. ST-LINK Utility installation location

- Wait until the installation is over.

Figure A.2.5. ST-LINK Utility installation

- When prompted by the driver installer, click on Next. When Windows asks for authorization to the driver installation, allow it.

Figure A.2.6. ST-LINK Driver installation Welcome window

Figure A.2.7. ST-LINK Driver setup authorization

- Wait until the driver is installed, then click on Finish.

Figure A.2.8. ST-LINK Driver installation Success window

- Click on Finish to exit the ST-LINK Utility Installation wizard.

Figure A.2.9. ST-LINK Utility installation Success window

A.3 – Text Editor Installation

- Execute Sublime Text 2 installer as Administrator.

Figure A.3.1. Running Sublime Text 2 installer

- In the Welcome window click on Next.

Figure A.3.2. Sublime Text 2 Installer Welcome window

- In the Installer Directory Window, select the default route.

Figure A.3.3. Sublime Text 2 installation Location window

- Tick the Add to explorer context menu option.

Figure A.3.4. Sublime Text 2 installation Additional Tasks window

- Once the installer is ready to install, click on the Install button.

Figure A.3.5. Sublime Text 2 installation Ready to Install window

- Wait until the installation is over. Then click on the Finish button.

Figure A.3.5. Sublime Text 2 installation Success window

- Open the Sublime Text 2 program. Go to Preferences  Color Scheme and select the option of your preference. You can disable the recent file history by setting Preferences  Settings-Default  "hot_exit" and Preferences  Settings-Default  "remember_open_files" to false.

Figure A.3.6. Sublime Text 2 Color Schemes

Figure A.3.7. Sublime Text 2 Settings

- KEIL-MDK PROJECTS

Objective: Learn how to create Keil-MDK projects that will allow the microcontroller programming.

Instructions: Read and follow the steps in the following subsections.

B.1 – Project Creation

- Open the Keil-MDK program. Go to Project  New µVision Project.

Figure B.1.1. Creating a new project

- Create a new folder named F103_Template and on it create the new project with the name F103_Template.

Figure B.1.2. Location of the new project

- Select the following device: STMicroelectronics  STM32F1 Series  STM32F103  STM32F103C8.

Figure B.1.3. Selecting target of new project

- Tick the options CMSIS  CORE and Device  Startup.

Figure B.1.4. Adding startup libraries to new project

- Left-click and then release Target 1. Change its name to F103_Template.

Figure B.1.5. Renaming the new project

- Expand the project’s root folder and rename the folder Source Group 1 to Source. Then close the Keil-MDK program.

Figure B.1.6. Renaming Source Group

- Make sure you can view file extensions by ticking File name extensions option in the View tab at the top of the file explorer.

Figure B.1.7. Enabling file name extensions in Windows Explorer

- Delete the DebugConfig\, Listings\, Objects\ folders and the F103_Template.uvguix.<USER> file. Then create a Libraries\, an Output\ and Simulation\ folders. Additionally, create a file named main.c. For this last file right-click on the background of the explorer window and then follow New  Text Document. Then rename the file to main.c. Open the file with the text editor. Substitute the content of Listing B.1.1 into this recently created file. Make sure the last line has an <ENTER>. If you installed the Sublime Text 2 editor, you can open the file by right clicking it and selecting Open with Sublime Text 2.

Figure B.1.8. Updated contents of new project folder

Listing B.1.1. Content of main.c file

#include <stm32f10x.h> #include "delay.h" int main( ) { // Initialization code while(1) { // Recurrent code } }

Figure B.1.9. Editing main.c file with text editor

- In the Libraries\ folder, create two folders named src\ and include\.

Figure B.1.10. Libraries\ folder contents

- Inside the src\ folder, create two files named _delay_ms.s and _delay_us.s. Open both files with the text editor and then replace them with the content of listings B.1.2 and B.1.3. Make sure the first line has a <TAB> and that the last line has an <ENTER>.

Figure B.1.11. src\ folder contents

Listing B.1.2. Content of _delay_ms.s file

AREA delay, CODE EXPORT _delay_ms ;make _delay_ms visible ALIGN _delay_ms PROC PUSH {r4, lr} outer MOV r1, #10282 inner SUBS r1, r1, #1 BNE inner SUBS r0, r0, #1 BNE outer POP {r4, pc} ENDP END

Figure B.1.12. Editing _delay_ms.s file with text editor

Listing B.1.3. Content of _delay_us.s file

AREA delay, CODE EXPORT _delay_us ;make _delay_us visible ALIGN _delay_us PROC PUSH {r4, lr} outer MOV r1, #4 inner SUBS r1, r1, #1 BNE inner SUBS r0, r0, #1 BNE outer POP {r4, pc} ENDP END

- Inside the include\ folder, create a file named delay.h. Open the file with the text editor. Substitute the content of Listing B.1.4 into this recently created file. Make sure the last line has an <ENTER>.

Figure B.1.13. include\ folder contents

Listing B.1.4. Content of delay.h file

extern void _delay_ms(uint32_t ms); extern void _delay_us(uint32_t ms);

Figure B.1.14. Editing delay.h file with text editor

- Open the file F103_Template.uvprojx. Right-click on the project’s name and select Options for Target ‘F103 Template’.

Figure B.1.15. Opening project’s options menu

- In the Target  Xtal (MHz): set a value of 72.0.

Figure B.1.16. Changing Xtal frequency

- Click on Output tab. Tick the Create HEX file option. In the same tab click on Select Folder for Objects and make it point to the Output\ folder inside the project’s root folder. Repeat for Listing tab  Select Folder for Listings.

| Figure B.1.17. Changing location for Objects | Figure B.1.18. Changing location for Listings |
| --- | --- |

- In the C/C++ tab  Include Paths parameter, add the .\Libraries\include folder.

Figure B.1.19. Adding project’s include folder parameter

- In the Debug tab  Use, select ST-Link Debugger as parameter. In the Utilities tab, check that the Use Debug Driver box is marked and then return to the Debug tab.

Figure B.1.20. Specifying project’s debugger

- Click on the Settings button. Then go to the Flash Download tab. Tick the Program, Verify and Reset and Run boxes. Select the Erase Full Chip option. After that, click on the Add button. Select the STM32F10x Flash Options and then on then click on the Add button. Finally click on the OK button in all the open windows of the project’s options.

Figure B.1.21. Flash Download options

Figure B.1.22. Flash Programming algorithms

- Right-click on the Source folder in the project’s tree. Select Add Existing Files to Group ‘Source’. Select (in this order) the main.c file in the project’s root folder and then all the files in the project’s Libraries\src\ folder. The project’s tree should show a structure similar to the one in Figure B.1.24.

| Figure B.1.23. Adding files to the ‘Source’ group | Figure B.1.24. Project tree after files addition |
| --- | --- |

- Click on the Rebuild button in the upper toolbar. In the Console window (at the bottom of the Keil program) you should get an output like the one in Figure B.1.26.

| Figure B.1.25. Rebuilding project | Figure B.1.26. Console output after rebuilding project |
| --- | --- |

- Inside the Simulation\ folder, create a Proteus schematic file named F103_Template.pdsprj that contains the circuit in Figure B.1.27. The terminal with an arrow is found in Right-click  Terminals  Power.

Figure B.1.27. Proteus circuit for project template

- Make sure that the STM32F103C6 component should has as properties the parameters depicted in Figure B.1.28. To edit the hidden pins, click on the Hidden Pins button. Once done, save the file and then close the program.

Figure B.1.28. STM32F103C6 Component Properties

- Erase the following:

- All the contents inside the Output\ folder.

- All the contents inside the Simulation\ folder except the F103_Template.pdsprj file.

- The F103_Template.uvguix.<USER> file.

You should get a result similar like the one depicted in Figure B.1.29. Finally, compress all the contents in the folder into a file named F103_Template.zip. Backup this file for future use (unless you wish to repeat all the steps of this section per each project you upload to the microcontroller).

Figure B.1.29. F103_Template folder

B.2 – Project Renaming

- In the following steps section, the new name will be L2. Decompress the F103_Template.zip file. Rename the folder as L2. Open the folder and rename the .uvprojx and .uvoptx files.

Figure B.2.1. F103_Template folder renaming

- Open the L2.uvprojx file and then rename the project’s folder as L2. To do this, remember to left-click once and then release the name aside the folder you want to rename.

Figure B.2.2. Project folder renaming

- Right-click on the L2 folder and then go to Options for Target ‘L2’  Output tab  Name of Executable and rename it as L2. Click on OK and then rebuild the whole project.

Figure B.2.3. Executable renaming

- Go to the Simulation folder and rename the F103_Template.pdsprj to L2.pdsprj.

Figure B.2.4. Simulation file renaming

- Open the simulation file and change the microcontroller program file to ..\Output\L2.hex. Click on OK and finally, save the file.

Figure B.2.5. Indicating to the microcontroller part the new .hex file

NOTE: The procedure in this sub-section should be executed each time a new program is designed for the microcontroller.

- KEIL-MDK DEBUGGER

Objective: Learn how to operate the Keil-MDK debugger.

Instructions: Read and follow the steps in the following subsections.

Prerequisites: Fulfilled Sections A and B.1 of this document.

C.1 – Debugger Usage

- According to their pinout, there exist mainly two types of ST-LINK V2 Debugger in the market. The red one in Figure C.1.1 will be named as Type A and the one in black one in Figure C.1.2 will be named as Type B. The main difference is on the SWDIO, SWIM and GND pin locations. Checkout which type do you have.

Figure C.1.1. ST-LINK V2 Debugger Type A

Figure C.1.2. ST-LINK V2 Debugger Type B

- Connect the ST-LINK V2 Debugger to the STM32F103 board according to the type you have and the following figures. Then connect the debugger to the computer. If everything is correct, the computer should display a message similar to the one displayed in Figure C.1.3. In the following labs, this will be the initial basic connection. WARNING: The debugger pinout may be different to those in the figures. Still, it should contain the same pins, so modify the connection accordingly. Ask help from your instructor should you require it. Connecting the debugger to the board in the wrong way might damage both components permanently.

|  |  | Figure C.1.5. Type A Physical connection |
| --- | --- | --- |
|  |  | Figure C.1.6. Type B Physical connection |
| Figure C.1.3. Type A Interface schematic | Figure C.1.4. Type B Interface schematic |  |

Figure C.1.7. Debugger driver successfully installed

- Rename the F3_Template project to LAB2A (check section B.2). Copy and paste the source code showed in Listing C.1.1 into the main.c file in the project then save it. The computer must have established communication with the STM32F103 board.

Listing C.1.1. LAB #2 First Program

#include <stm32f10x.h> const uint16_t A[3][3] = { { 1,3,5 }, { 5,1,3 }, { 3,5,1 } }; const uint16_t B[3][3] = { { 2,4,6 }, { 6,2,4 }, { 4,6,2 } }; int main( void ) { __IO uint16_t C[3][3] = { { 0,0,0 }, { 0,0,0 }, { 0,0,0 } }; __IO uint16_t i,j,k,accum; for( i=0; i<3; i++ ) { for( j=0; j<3; j++ ) { accum = 0; for( k=0; k<3; k++ ) { accum += A[i][k]*B[k][j]; } C[i][j] = accum; } } for(;;); }

- Go to Project  Rebuild All Target Files (or press the  icon in the toolbar).

- Go to Debug  Start Debug Session (or press <CTRL> + <F5> or press the   icon in the toolbar).

- Go to View  Watch Windows  Watch 1 (or search for the Watch windows icon). Click on it.

Figure C.1.8. Opening the Watch windows

- In the cell Enter Expression from the Watch 1 window, type A. With this, we have added the A matrix to the Watch 1 window. Repeat this step for matrices B and C.

- Right-click on each of the variables in the Watch 1 window and uncheck the Hexadecimal Display option. Then expand all the variables. You should get an output similar like the one displayed in Figure C.1.9.

Figure C.1.9. Matrix variables in Watch 1 Window

- Go to line 11 in the code. Right-click on it, and select Insert/Remove Breakpoint (or press F9). A red dot should appear on the line number of that line, like the one in Figure C.1.11.

Figure C.1.10. Insert/Remove Breakpoint option

Figure C.1.11. Placing a breakpoint

- Go to Debug  Run (or use the shortcut F5 or press the    icon in the toolbar).  The program should now stop in line 11 and the values in the Watch 1 window should be like in Figure C.1.9.

- If the values are displayed in a matrix form, then, the outputs of Watch 1 window are:

| Matrix A | Matrix B | Matrix C |
| --- | --- | --- |
| 135513351 | 246624462 | 000000000 |

- Remove the breakpoint in Line 11 by repeating step 8 or by following Debug  Kill All Breakpoints. Place another breakpoint in Line 13. Press the <F5> key twice. The values of matrix C should be the following:

Matrix C

404028000000

- Press the <F5> key two more times. The values of matrix C should be the ones depicted in Table C.1.1:

Table C.1.1. Representing matrix C after two debug steps in a table

| Step | Matrix C |
| --- | --- |
| 2 | 404028284040000 |
| 3 | 404028284040402840 |

- Remove the breakpoint in line 13 (right-click on the number line and select again Insert/Remove Breakpoint). Create a new one in line 15. Click on the Reset button to reset the CPU.

Figure C.1.12. Resetting the CPU

- Press the <F5> key. The program should stop at line 15.

- Add the accum variable into the Watch 1 window. Disable the Hexadecimal Display option.

- Press the <F5> key twice. In the C and accum variables from the Watch 1 window and make it display decimal values, you should get something similar to Figure C.1.13.

Figure C.1.13. Updating variables

- In another representation, the last results are expressed as in Table C.1.2.

Table C.1.2. Representing variables in Watch 1 window in a table

| Step | accum | Matrix C |
| --- | --- | --- |
| 1 | 40 | 4000000000 |

- (R) Press the <F5> key until the matrix C is completely calculated. For each step, add a row to Table C.1.2 and fill the information requested in the columns.

Step; accum; Matrix C. 1; 40; 4000000000. 2; 40; 40400000000. 3; 28; 404028000000. 4; 28; 4040282800000. 5; 40; 40402828400000. 6; 40; 404028284040000. 7; 40; 4040282840404000. 8; 28; 40402828404040280. 9; 40; 404028284040402840.

- Remove the breakpoint in Line 15. Place a new one in Line 18. Reset the debugger. Compress the variables A and B in the Watch 1 window (click on the – icon for each variable). Remove the variable accum in the Watch 1 window (right-click  Remove Watch accum). Add the variables (in this order) i, j, k, A[i][k], B[k][j] and accum. Remove on each variable the Hexadecimal Display option. Then press the <F5> key. The Watch 1 window should display something like Figure C.1.14. for the involved variables:

Figure C.1.14. Updating variables

- In another representation, the last results are expressed in Table C.1.3:

Table C.1.3. Representing variables in figure C.1.10 in a table

| Step | Matrix C | i | j | k | A[i][k] | B[k][j] | accum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 000000000 | 0 | 0 | 0 | 1 | 2 | 0 |

- (R) Press the <F5> key until the matrix C is completely calculated. For each step, add a row to Table C.1.3 and fill the information requested in the columns. Copy and paste the full table in the red box.

0

0

0Step; Matrix C; i; j; k; A[i][k]; B[k][j]; Accum. 0; 000000000; 0; 0; 0; 1; 2; 0. 1; 000000000; 0; 0; 1; 3; 6; 2. 2; 000000000; 0; 0; 2; 5; 4; 20. 3; 4000000000; 0; 1; 0; 1; 4; 0. 4; 4000000000; 0; 1; 1; 3; 2; 4. 5; 4000000000; 0; 1; 2; 5; 6; 10. 6; 40400000000; 0; 2; 0; 1; 6; 0. 7; 40400000000; 0; 2; 1; 3; 4; 6. 8; 40400000000; 0; 2; 2; 5; 2; 18. 9; 404028000000; 1; 0; 0; 5; 2; 0. 10; 404028000000; 1; 0; 1; 1; 6; 10. 11; 404028000000; 1; 0; 2; 3; 4; 16. 12; 4040282800000; 1; 1; 0; 5; 4; 0. 13; 4040282800000; 1; 1; 1; 1; 2; 20. 14; 4040282800000; 1; 1; 2; 3; 6; 22. 15; 40402828400000; 1; 2; 0; 5; 6; 0. 16; 40402828400000; 1; 2; 1; 1; 4; 30. 17; 40402828400000; 1; 2; 2; 3; 2; 34. 18; 404028284040000; 2; 0; 0; 3; 2; 0. 19; 404028284040000; 2; 0; 1; 5; 6; 6. 20; 404028284040000; 2; 0; 2; 1; 4; 36. 21; 4040282840404000; 2; 1; 0; 3; 4; 0. 22; 4040282840404000; 2; 1; 1; 5; 2; 12. 23; 4040282840404000; 2; 1; 2; 1; 6; 22. 24; 40402828404040280; 2; 2; 0; 3; 6; 0. 25; 40402828404040280; 2; 2; 1; 5; 4; 18. 26; 40402828404040280; 2; 2; 2; 1; 2; 38. 27; 404028284040402840; 3; 3; 3; 6; 8192; 40.

- (R) In the following red box, place the commented main.c program used in Listing C.1.1.

#include <stm32f10x.h> //Se trata de un código de multiplicación de matrices que utiliza el método //convencional para lograr esta tarea. //para poder comprenderlo mejor, es necesario comenzar desde la linea con: "for( k=0; k<3; k++ )" const uint16_t A[3][3] = { { 1,3,5 }, { 5,1,3 }, { 3,5,1 } }; const uint16_t B[3][3] = { { 2,4,6 }, { 6,2,4 }, { 4,6,2 } }; int main( void ) { __IO uint16_t C[3][3] = { { 0,0,0 }, { 0,0,0 }, { 0,0,0 } }; __IO uint16_t i,j,k,accum; //Se hace un ciclo que se repetirá 3 veces acumulando las columnas de C for( i=0; i<3; i++ ) { for( j=0; j<3; j++ ){ accum = 0; //Se hace un ciclo que se repetirá 3 veces realizando la multiplicación que habrá en cada espacio de C for( k=0; k<3; k++ ){ //Se realiza la multiplicación entre cada elemento de la matríz. accum += A[i][k]*B[k][j]; } //Se hace multiplicando las filas de A por las columnas de B. Se van sumando los resultados //En esta línea se cóloca en la celda respectiva el resultado de la fila A con la columna B (ya sumados) C[i][j] = accum; } } //El ciclo principal permite pasar a los resultados de las filas que deberán colocarse en C for(;;); }

C.2 – Debugging Configuration registers

- Connect the ST-LINK Debugger to the STM32F103 board according to section C.1.

- Rename the F3_Template project to LAB2B (check section B.2). Copy and paste the source code showed in Listing C.2.1 into the main.c file in the project then save it. The computer must have established communication with the STM32F103 board.

Listing C.2.1. LAB #2 Second Program

#include <stm32f10x.h> #include "delay.h" int main( ) { RCC->APB2ENR ¦= RCC_APB2ENR_IOPCEN; GPIOC->CRH = ( GPIOC->CRH & ~GPIO_CRH_CNF13 & ~GPIO_CRH_MODE13 ) ¦ GPIO_CRH_MODE13; while(1) { GPIOC->ODR ^= 1<<13; _delay_ms(500); } }

- Rebuild all the target files of the project and open the debugger. Follow name the F3_Template project to LAB2B (check section B.2). Copy and paste the source code showed in Listing C.2.1 into the main.c file in the project. The computer must have established communication with the STM32F103 board.

- Place a breakpoint in Line 8. If you follow Peripherals  System Viewer  GPIO  GPIOC, a window like in Figure C.2.1 appears at the right side of the application. This window shows the data in the microcontroller registers. Each time the debugger is stopped, the window contents are refreshed.

| Figure C.2.1. Accessing peripheral registers | Figure C.2.2. Peripheral registers contents |
| --- | --- |

- (R) Debug the program. In the following red box, report the final content of the three peripheral registers involved in it.

- (R) In the following red box, place a screenshot of a Proteus simulation running the program in Listing C.2.1.

- (R) In the following red box, place the commented main.c program used in Listing C.2.1.

#include <stm32f10x.h> #include "delay.h" int main( ) { // Declaración de activaciónde Clock en pines de entrada RCC->APB2ENR ¦= RCC_APB2ENR_IOPCEN; //Declaración de PinC13 como entrada Pull up GPIOC->CRH = ( GPIOC->CRH & ~GPIO_CRH_CNF13 & ~GPIO_CRH_MODE13 ) ¦ GPIO_CRH_MODE13; while(1) { //Hacer lectura del estado del pin cada 500 milisegundos GPIOC->ODR ^= 1<<13; //Espera de 500 ms _delay_ms(500); } } }